Differences in Silvertone 1472 and 1482 Circuits
Notes on Airline 9013A which is very similar to the 1472
I did a little more ground work in prep for bklynboy doing the 1482 layout. I went over both 1472 and 1482 schematics and then compared the differences. I also looked at my 1482 circuit to verify the differences found on the schematics and the values of components in place on my amp. This is what I found:
1- Power Tube cathode bypass cap and cathode resistor:
a- Cathode Bypass Cap-The 1472 schematic shows 10uf and 1482 shows 22uF. I checked my amp and it had a 25uf/25V in place.
· Airline – Power tube bypass cap is 40uf/25V

b- Cathode Resistor-The 1472 schematic shows 220 Ohm/5 watt and 1482 shows 270 Ohm/5 watt. My 1482 has a 270 Ohm/5 watt in place.
· Airline is 270 Ohm/5 watt
2- In the Tremolo circuit:
a- R26 is 2 Meg on the 1472 schematic and on the 1482 is 2.2 Meg. I checked my circuit and it was 2.2 Meg in place.
· Airline is 2M – but these values are within tolerance
b- C14 is .05 uF on 1472 Schematic and on 1482 shows .047 uF. I checked my circuit and was .05 uF in place.
· Airline is .05 – again no practical difference
3- Power Supply/Power Transformer:
a- 2nd Power Node Resistor-R35 on 1472 schematic is 15K and on 1482 is 27K. I checked my circuit and it is a 27K in place.
· Airline is 15K

· Airline and 1472 have an additional node with a 100K resistor and a 0.1 capacitor to ground. Some term this cap a filter cap and some term it a smoothing cap. Some Valco amps have a similar power rail.
· Airline and 1472 both have a lower secondary voltage PT

b- 1472 had an artificial center tap Heater Grounding-with R37 and R38 68 Ohm resistors to ground.
· Airline has an unusual artificial center tap heater circuit with one leg being the pilot lamp and the opposite leg a 3.3 Ohm resistor. The total heater voltage is correct at 6.3v, but the two legs are very unbalanced.

c-1482 has dedicated Center tap Ground-The 1482 schematic has a red/yellow heater ground. I checked my circuit and the heater red/yellow ground is present.
· Airline – the red/yellow wire is the PT main center tap. 
d-If you look a the view with PT, the green wire to the Cap Can ground is the main PT ground and the red/yellow wire running to another Can Cap Ground is the heater ground wire. Of course the other orange wire over the top of the red/yellow heater ground is screen voltage after the first 2.2K power node resistor.
· Airline - There are no green wires exiting the PT – Heater wires are yellow
Other – the 1472 and 1482 have 1 Mic channel and 1 Instrument channel – the Airline has 2 identical channels each with 2 input jacks with a 68K grid stopper. They are not wired hi/low.

And – the Airline has the chassis at the top where it belongs, and much cooler knobs.

THIS WELCOME ANYONE TO DOUBLE CHECK BEHIND ME AND SEE IF YOU SEE ANYTHING ELSE. PLATEFIRE
